Purpose

The purpose for this circuit is to be a cheap alternative to burglary alarm systems. It uses very cheap parts like PIR Sensors, 555 timers, Piezos and capacitors that not only alerts if an intruder through a loud sound, but also will send a text message to your phone. There will be no monthly charge, just a one time price of around $100 if it were to be sold.

Function

It is supposed to detect heat signatures and movement through the PIR sensor . There is a delay created through a 555 timer and a capacitor to give users a cooldown. The PIR sensor once it detects movement, will send a signal to the Raspberry Pi which activities the script and texts the user that someone is there and also makes the Pi Camera takes a picture and also send the picture through email. It will also send a signal to a Piezo and make a sound.  This process will repeat every minute.

Procedure

- Setup Raspberry Pi

  - Install operating system using SD card

  - Input code to handle PIR Sensor Output

- Setup PIR sensor on breadboard

  - Put in LED

  - Put in Piezo

  - Connect power rails from second breadboard and connect respective resistors

  - Hook up ground from Raspberry Pi

  - Hook up batteries

- Setup timer delay for alarm

  - Get 3.3 V or 5V power  and ground rail from Raspberry Pi

  - Setup timer and resistors

  - Hook up ‘armed’ LED

  - Connect output to positive rail on primary breadboard

- Program the code for the emailing and text messaging of intruder.

  - Code the texting function that alerts users through text that someone is in the vicinity

  - Obtain a Pi Camera and hook it up to the Raspberry Pi

  - Code the part of the program that takes a picture using the Pi camera and emails users of the picture taken

  - Combain the scripts

- Add to script so that it ignores background voltage so that the script does not randomly turned on using ‘event_detect()’.

Parts List

| Component | Number | Use |
| --- | --- | --- |
| PIR Sensor | 1 | To identify the movement of objects around the vicinity and produce an output |
| AA Batteries | 3 | The power source for the project for testing |
| Battery Holder *Capacity for 3 AA batteries with breadboard wires and pins | 1 | To connect batteries to breadboard for testing |
| Breadboard Piezo Buzzer | 1 | Acts as the alarm |
| Polarized Capacitor (100 uF, 16 V) | 1 | Helps in creating delay |
| 555 timer | 1 | Along with the capacitor brings the delay |
| 47 kilo ohm resistor | 1 | Also to cause delay |
| 470 ohm resistor | 1 | Also to cause delay |
| Switch | 2 | To control the circuit |
| LED | 3 | Lighting System |
| Jumper Wires | 20 -30 | Carry current through breadboard |
| Breadboard | 2 | Where all the parts are located |
| Raspberry Pi Model 3 B | 1 | *I will get this myself! |
| Pi Camera | 1 | Takes the picture that will be emailed to the user. |

How It Should Look

Issues I Faced

| Problem | Solution |
| --- | --- |
| Using the Raspberry Pi’s power rails became problematic because the 3.3V was a bit too much for the PIR sensor. Though resistors could be used, it was simpler to use an external power source | - Used 3 x 1.5V AA Batteries as Breadboard power source |
| Raspberry Pi would not boot, the operating system would not get burned onto the SD card | - It turns out the SD card was improper to the point where even reformatting it could not fix it, however, getting a new SD card seemed to work |
| Raspberry Pi camera was not working | - It turns out it was not plugged in correctly and a command ‘vcgencmd get_camera’ had to be used |
| Raspberry Pi was detecting random environmental voltage so it would run my script while not voltage was being outputted by my PIR sensor | - Within the script, instead, by checking for voltage dips, the script worked |

Future Plans

- I plan on making the design a little bit more simplified, perhaps by instead using a Pi Zero and a smaller breadboard. This way, by having it more compact it will be a lot easier to use.

- I also plan on making it so that the Piezo makes a much louder sound, this way it is easier for users to hear the sound

- I also plan on adding buttons/more user input to turn it on and off and help set a delay, this would make it more user friendly

Conclusion

I chose this project to create a device that would be an alternative to standard security systems, be cheaper than them, but also provide much more functionality. I have to say, this truly has been very successful, in terms of wiring, coding and the end product. Everything that I planned on implementing, especially the emails and text messages worked out in the end. There are a few things I wanted to add, including added simplicity. However, as a beginner project, it turned out pretty well.

Pi Security

Security For Everyone!

By: Jarif Aftab

Mr. Knowles

TEJ3UO

June 14th, 2018